SastaDrive

3-Phase FOC Motor Driver

Electrical Specifications & User Reference

Rev 1  |  April 2026  |  MEKATRA

# 1. Overview

SastaDrive is a compact, low-cost, 3-phase Field-Oriented Control (FOC) capable motor driver designed for custom BLDC and PMSM motors. It integrates power stage, gate drive, current sensing, bus voltage sensing, and an STM32G431CBT6 microcontroller on a single 100 x 100 mm 2-layer PCB. The board is designed for RnD and small-volume applications requiring smooth, precise torque delivery at low to mid RPM.

Key Features

- 24 V nominal DC bus input, 3-phase BLDC/PMSM output

- FOC-capable with 3-phase inline current sensing (INA181A2 + 20 mOhm shunts)

- STM32G431CBT6 MCU with hardware complementary PWM, dual simultaneous ADC

- UCC27710 half-bridge gate drivers with bootstrap supply

- 24 V to 12 V onboard buck converter (TPS54334)

- I2C encoder header for AS5600 magnetic encoder module

- SWD debug header + UART header for firmware development

- Aluminium frame compatible — bottom copper pour acts as heatsink interface

# 2. Absolute Maximum Ratings

WARNING — Exceeding any value in this table may permanently damage the device. These are stress ratings only, not operating conditions. The board should never be operated at absolute maximum values.

| Parameter | Rating | Unit | Note |
| --- | --- | --- | --- |
| DC Bus Supply Voltage (VIN) | 28 | V DC | Transient spikes above 24 V. TVS clamps at 26 V. |
| DC Bus Voltage — Absolute Peak | 30 | V | Including inductive kickback. Do not exceed. |
| Phase Output Current — Peak (< 1 ms) | 10 | A | Limited by winding thermal, not FET |
| Phase Output Current — Continuous | 5 | A RMS | Derate above 40 C ambient |
| MOSFET Vds | 40 | V | IPZ40N04S5L2R8 rating |
| Gate Drive Supply (VCC to GND) | 20 | V | UCC27710 VCC pin max |
| Logic Supply Voltage (3.3 V rail) | 3.6 | V | STM32 VDDA absolute max |
| MCU GPIO Input Voltage | 3.6 | V | With respect to GND; do not drive 5 V signals |
| INA181 Common Mode Input | 80 | V | Not applicable at 24 V bus but noted |
| INA181 Supply Voltage | 5.5 | V | Powered from 3.3 V rail; well within limit |
| Operating Temperature — PCB Surface | 85 | C | STM32 industrial grade limit |
| Storage Temperature | 125 | C | -40 C minimum |
| ESD (HBM) on signal pins | 2000 | V | Standard HBM; avoid touching pins without grounding |

# 3. Recommended Operating Conditions

All specifications below are guaranteed only when operating within these conditions. Values outside this range are not characterized.

| Parameter | Min | Typ | Max | Unit | Conditions |
| --- | --- | --- | --- | --- | --- |
| DC Bus Supply Voltage | 18 | 24 | 26 | V | Steady state; protected by TVS SMBJ26A |
| DC Bus Supply Current | - | - | 10 | A | Peak; depends on motor load |
| Standby Power Consumption | - | 180 | - | mW | No motor drive, MCU active, 24 V input |
| PWM Frequency | 10 | 20 | 40 | kHz | Centre-aligned; set in TIM1 |
| Dead Time (hardware) | 100 | 200 | 800 | ns | Configured in STM32 TIM1 BDTR register |
| Ambient Operating Temperature | -20 | 25 | 65 | C | Derate phase current above 40 C |
| Humidity (non-condensing) | 5 | - | 85 | %RH | No conformal coating on Rev 1 |

# 4. Power Supply Specifications

## 4.1 DC Bus Input

| Parameter | Min | Typ | Max | Unit | Conditions |
| --- | --- | --- | --- | --- | --- |
| Input Voltage | 18 | 24 | 26 | V | Fused externally; board has no fuse |
| Input Capacitance (bulk) | - | 940 | - | uF | 2x 470 uF 35 V electrolytic on board |
| TVS Clamp Voltage | 26 | 28.9 | 31.5 | V | SMBJ26A at 1 A, see datasheet |
| Reverse Polarity Protection | — | None | — | - | No protection; use external diode if needed |

## 4.2 12 V Rail (Gate Driver Supply)

| Parameter | Min | Typ | Max | Unit | Conditions |
| --- | --- | --- | --- | --- | --- |
| Output Voltage | 11.4 | 12 | 12.6 | V | TPS54334; includes regulation accuracy |
| Output Current — Max | - | - | 3 | A | TPS54334 rated; gate drive draws < 100 mA typical |
| Switching Frequency | - | 570 | - | kHz | TPS54334 fixed internal |
| Ripple Voltage (pk-pk) | - | 50 | 120 | mV | At full load; measured at output cap |
| Input (from 24 V bus) | 18 | 24 | 28 | V | TPS54334 VIN max 28 V — see caution |

## 4.3 3.3 V Rail (MCU and Sensor Supply)

| Parameter | Min | Typ | Max | Unit | Conditions |
| --- | --- | --- | --- | --- | --- |
| Output Voltage | 3.234 | 3.3 | 3.366 | V | MCP1700T LDO from 5 V; includes load regulation |
| Output Current — Max | - | - | 250 | mA | MCP1700T limit; board draws < 100 mA typical |
| Ripple Voltage (pk-pk) | - | 10 | 30 | mV | After ferrite bead filter |
| VDDA Reference (VREF+) | 3.290 | 3.300 | 3.310 | V | REF3033 precision reference for ADC |

# 5. Power Stage Specifications

## 5.1 MOSFET — IPZ40N04S5L2R8ATMA1

| Parameter | Min | Typ | Max | Unit | Conditions |
| --- | --- | --- | --- | --- | --- |
| Drain-Source Voltage (Vds) | - | - | 40 | V | Do not exceed; bus is 24 V nominal |
| Continuous Drain Current | - | - | 40 | A | Package limited; application limited to 10 A peak |
| Rds(on) at 10 V Vgs | - | 2.0 | 2.8 | mOhm | At 25 C; increases at higher temperature |
| Rds(on) at 4.5 V Vgs | - | 3.2 | 4.5 | mOhm | Relevant during bootstrap charging transient |
| Gate Threshold Voltage | 1.2 | 1.9 | 2.5 | V | Logic level; fully on at 10 V Vgs |
| Gate Charge (Qg) | - | 15 | 20 | nC | At 10 V Vgs, 20 V Vds |
| Turn-On Delay (td_on) | - | 5 | - | ns | With 10 Ohm gate resistor |
| Turn-Off Delay (td_off) | - | 8 | - | ns | With 10 Ohm gate resistor |
| Thermal Resistance Rth(j-c) | - | 1.5 | - | C/W | Junction to case; mount to Cu pour |

## 5.2 Gate Driver — UCC27710DR

| Parameter | Min | Typ | Max | Unit | Conditions |
| --- | --- | --- | --- | --- | --- |
| Supply Voltage (VDD/VCC) | 10 | 12 | 20 | V | Powered from 12 V rail on this board |
| Bootstrap Voltage (VBST-VS) | 10 | 11.3 | - | V | After SS14 diode drop from 12 V |
| High-Side Gate Drive (Vgs) | - | 10.9 | 11.5 | V | Bootstrap - phase node; adequate for FET |
| Low-Side Gate Drive (Vgs) | - | 11.3 | - | V | VCC - 0.7 V; adequate for FET |
| Output Current — Source | - | 0.5 | - | A | Peak gate source current |
| Output Current — Sink | - | 1.0 | - | A | Peak gate sink current; faster turn-off |
| Propagation Delay — HI | - | 40 | 65 | ns | Input to output |
| Propagation Delay — LI | - | 16 | 25 | ns | Input to output |
| Bootstrap Diode | - | - | - | - | SS14 Schottky external; 1 A, 40 V |
| Bootstrap Cap | 100 | 220 | 470 | nF | Per phase; 10 V ceramic minimum rating |

# 6. Current Sensing Specifications

## 6.1 Phase Current Sense (3-Phase Inline)

Each phase uses a 20 mOhm shunt resistor with an INA181A1 (gain = 20 V/V) current sense amplifier. REF pin is biased to VDDA/2 = 1.65 V enabling bidirectional current measurement. Software offset calibration must be performed at startup before enabling PWM.

| Parameter | Min | Typ | Max | Unit | Conditions |
| --- | --- | --- | --- | --- | --- |
| Shunt Resistance | 19.8 | 20 | 20.2 | mOhm | 1% tolerance; HOYLR2512-2W-20MR x1 per phase |
| Shunt Power Dissipation | - | 0.25 | 2.0 | W | At 3.5 A cont / 10 A peak; 2 W rated |
| Amplifier Gain | - | 20 | - | V/V | INA181A1 fixed gain |
| Measurable Current — Positive | 0 | - | +5 | A | ADC full scale = VDDA = 3.3 V |
| Measurable Current — Negative | -5 | - | 0 | A | REF pin biased to 1.65 V midpoint |
| Output at Zero Current | 1.60 | 1.65 | 1.70 | V | REF pin voltage; verify after assembly |
| Output at +5 A | 3.25 | 3.30 | 3.35 | V | Verify does not saturate ADC |
| Output at -5 A | 0.00 | 0.05 | 0.10 | V | Verify does not saturate at zero |
| Input Offset Voltage (Vos) | - | ±25 | ±150 | uV | INA181A1 at 25 C; corrected by calibration |
| Calibration Accuracy (post-cal | ±0.5 | - | ±2 | % | Software offset calibration at boot |
| Common Mode Voltage Range | 0 | - | 80 | V | INA181 spec; 24 V bus well within range |
| Bandwidth (-3 dB) | - | 350 | - | kHz | INA181A1; sufficient for 40 kHz PWM |
| ADC Resolution (12-bit, 3.3 V) | - | 0.806 | - | mA | 1 LSB = 0.8 mV / (20mR x 20) = 2 mA |

## 6.2 DC Bus Current Sensing

| Parameter | Min | Typ | Max | Unit | Conditions |
| --- | --- | --- | --- | --- | --- |
| Shunt Resistance | - | 20 | - | mOhm | Single 20 mOhm; unidirectional |
| Amplifier Gain | - | 20 | - | V/V | INA181A1 |
| Measurable Range | 0 | - | +10 | A | REF pin to GND; unidirectional only |
| Output at 10 A | - | 4.0 | - | V | Exceeds 3.3 V ADC — limit firmware to 8.25 A |

CAUTION — DC bus current sense output saturates at ~8.25 A with INA181A1 gain=20 and 20 mOhm shunt feeding a 3.3 V ADC. Implement firmware current limit at 7 A to ensure ADC is never saturated.

# 7. Voltage Sensing Specifications

## 7.1 DC Bus Voltage Sensing

| Parameter | Min | Typ | Max | Unit | Conditions |
| --- | --- | --- | --- | --- | --- |
| Divider Ratio | - | 0.091 | - | - | R1=100k, R2=10k; Vout = Vbus x 0.0909 |
| Output at 24 V Bus | - | 2.18 | - | V | To STM32 ADC; within 3.3 V range |
| Output at 28 V (max) | - | 2.54 | - | V | Still within ADC range; safe |
| ADC Resolution | - | 26.7 | - | mV/LSB | 12-bit ADC, 3.3 V ref: 11 mV/count on bus |
| Clamping | - | 3.3 | - | V | Zener diode on ADC pin; protects MCU |
| Input Impedance | - | 110 | - | kOhm | R1 + R2 in series; negligible bus loading |

# 8. Microcontroller Specifications

## 8.1 STM32G431CBT6

| Parameter | Min | Typ | Max | Unit | Conditions |
| --- | --- | --- | --- | --- | --- |
| Core | - | - | - | - | ARM Cortex-M4 with FPU, 170 MHz max |
| Operating Frequency | - | 170 | 170 | MHz | Configured to max for FOC performance |
| Flash Memory | - | 128 | - | KB | Sufficient for SimpleFOC + custom code |
| RAM | - | 32 | - | KB | - |
| Supply Voltage (VDD) | 1.71 | 3.3 | 3.6 | V | Powered from 3.3 V rail |
| ADC Channels Used | - | 6 | - | - | 3x phase I, bus V, bus I, NTC temp |
| ADC Resolution | - | 12 | - | bit | Dual ADC simultaneous triggered by TIM1 |
| ADC Sampling Rate | - | =PWM | - | - | Sampled at PWM valley per cycle |
| PWM Timer | - | TIM1 | - | - | Advanced timer; complementary outputs |
| PWM Resolution | - | 12 | - | bit | At 20 kHz; 170 MHz / (20k x 2) = 4250 counts |
| I2C (Encoder) | - | 400 | - | kHz | Fast mode; AS5600 on PB6/PB7 |
| UART (Debug) | - | 115200 | - | baud | USART3 on PB10/PB11 |
| GPIO — Logic Level | - | 3.3 | - | V | Not 5 V tolerant — do not connect 5 V signals |

# 9. Connector and Test Point Reference

## 9.1 Power Input Connector (J1 — XT30 or KF2510 2-Pin)

| Pin/Pad | Label | Type | Description |
| --- | --- | --- | --- |
| 1 | VIN+ | PWR | 24 V DC positive. Connect to fused supply. 18–26 V operating range. |
| 2 | GND | GND | Power ground. Connect first before signal connectors. |

## 9.2 Motor Phase Output (J2 — Bullet or KF2510 3-Pin)

| Pin/Pad | Label | Type | Description |
| --- | --- | --- | --- |
| 1 | PHASE_A | PWR | Motor phase A. Connect to motor phase wire. No polarity requirement — change motor direction in firmware. |
| 2 | PHASE_B | PWR | Motor phase B. |
| 3 | PHASE_C | PWR | Motor phase C. |

## 9.3 SWD Debug Header (J3 — 2.54mm 4-Pin)

| Pin/Pad | Label | Type | Description |
| --- | --- | --- | --- |
| 1 | SWDIO | DIO | SWD data line. Connect to ST-Link pin 7. |
| 2 | SWDCLK | DIO | SWD clock line. Connect to ST-Link pin 9. |
| 3 | 3.3V | PWR | 3.3 V output. Can power ST-Link VTref. Max 50 mA draw. |
| 4 | GND | GND | Ground. Connect to ST-Link pin 8. |

## 9.4 UART Debug Header (J4 — 2.54mm 3-Pin)

| Pin/Pad | Label | Type | Description |
| --- | --- | --- | --- |
| 1 | USART3_TX | DO | MCU transmit. Connect to USB-UART RX pin. 3.3 V logic — do not connect 5 V UART adapter directly. |
| 2 | USART3_RX | DI | MCU receive. Connect to USB-UART TX pin. 3.3 V logic only. |
| 3 | GND | GND | Common ground with USB-UART adapter. |

## 9.5 Encoder Header (J5 — 2.54mm 4-Pin, I2C)

| Pin/Pad | Label | Type | Description |
| --- | --- | --- | --- |
| 1 | 3.3V | PWR | 3.3 V supply for AS5600 module. Max 20 mA. |
| 2 | GND | GND | Ground. |
| 3 | SCL (PB6) | DIO | I2C clock. 4.7 kOhm pull-up to 3.3 V on board. 400 kHz fast mode. |
| 4 | SDA (PB7) | DIO | I2C data. 4.7 kOhm pull-up to 3.3 V on board. |

## 9.6 User Buttons

| Pin/Pad | Label | Type | Description |
| --- | --- | --- | --- |
| SW1 — BT1 | PB3 | DI | User button 1. Active LOW. Internal pull-up enabled in firmware. |
| SW2 — BT2 | PB4 | DI | User button 2. Active LOW. |
| SW3 — BT3 | PB5 | DI | User button 3. Active LOW. |
| SW4 — BOOT | BOOT0 | DI | Hold LOW on power-up for normal boot. HIGH = DFU bootloader mode. |
| SW5 — NRST | NRST | DI | System reset. Active LOW momentary press resets MCU. |

# 10. MCU Signal Pin Assignment

Complete STM32G431CBT6 pin assignment for SastaDrive Rev 1. All unused GPIO pins are left floating — do not connect external signals to unlisted pins without consulting schematic.

| MCU Pin | Net Name | Peripheral | Direction | Description |
| --- | --- | --- | --- | --- |
| PA0 | PHASE_A_CSENSE | ADC1_IN1 | AI | Phase A inline current sense output from INA181 |
| PA1 | PHASE_B_CSENSE | ADC1_IN2 | AI | Phase B inline current sense output from INA181 |
| PA2 | PHASE_C_CSENSE | ADC2_IN3 | AI | Phase C inline current sense output from INA181 |
| PA3 | DCBUS_VSENSE | ADC1_IN4 | AI | DC bus voltage via 100k/10k divider + zener clamp |
| PA4 | INPUT_CSENSE | ADC2_IN17 | AI | DC bus current sense output from INA181 |
| PA5 | NTC_TEMP | ADC1_IN13 | AI | NTC thermistor divider — power stage temperature |
| PA7 | A_LID (CH1N) | TIM1_CH1N | DO | Phase A low-side complementary PWM output |
| PA8 | A_HID (CH1) | TIM1_CH1 | DO | Phase A high-side PWM output |
| PA9 | B_HID (CH2) | TIM1_CH2 | DO | Phase B high-side PWM output |
| PA10 | C_HID (CH3) | TIM1_CH3 | DO | Phase C high-side PWM output |
| PA13 | SWDIO | SYS_SWDIO | DIO | SWD debug data — do not use as GPIO |
| PA14 | SWDCLK | SYS_SWDCLK | DIO | SWD debug clock — do not use as GPIO |
| PB0 | B_LID (CH2N) | TIM1_CH2N | DO | Phase B low-side complementary PWM output |
| PB1 | C_LID (CH3N) | TIM1_CH3N | DO | Phase C low-side complementary PWM output |
| PB3 | BT1 | GPIO_IN | DI | User button 1, active LOW, internal pull-up |
| PB4 | BT2 | GPIO_IN | DI | User button 2, active LOW, internal pull-up |
| PB5 | BT3 | GPIO_IN | DI | User button 3, active LOW, internal pull-up |
| PB6 | ENCODER_SCL | I2C1_SCL | DIO | I2C1 clock — AS5600 encoder; 4.7 kOhm pull-up on board |
| PB7 | ENCODER_SDA | I2C1_SDA | DIO | I2C1 data — AS5600 encoder; 4.7 kOhm pull-up on board |
| PB10 | USART3_TX | USART3_TX | DO | UART transmit — 3.3 V logic only |
| PB11 | USART3_RX | USART3_RX | DI | UART receive — 3.3 V logic only |
| PC13 | LED1 | GPIO_OUT | DO | Status LED 1 — active HIGH |
| PC14 | LED2 | GPIO_OUT | DO | Status LED 2 — active HIGH |
| PC15 | LED3 | GPIO_OUT | DO | Status LED 3 — active HIGH |

# 11. Safety and Operating Guidelines

DANGER — This board operates at voltages and currents that can cause severe burns, fire, or permanent equipment damage. Always discharge bus capacitors before handling. Verify all connections before applying power.

## 11.1 Before First Power-On Checklist

- Verify input polarity with multimeter before connecting power supply

- Confirm 24 V supply current limit is set to 2 A for first power-on

- Check for solder bridges across MOSFET pins using magnification

- Verify 12 V rail present at gate driver VCC pins before enabling PWM

- Verify 3.3 V rail present at MCU VDD pins before programming

- Confirm BOOT0 is pulled LOW for normal operation (not bootloader)

- Do not connect motor until firmware current limits are configured

- Run current sense calibration routine before enabling any PWM output

## 11.2 Motor Connection

- Motor phase wires can be connected in any order — change direction in firmware, not by swapping wires while powered

- Never disconnect motor phases while PWM is active — induces high voltage spikes

- Encoder must be connected and reading valid data before enabling FOC closed-loop

- AS5600 requires a diametrically magnetised 6 mm disc magnet on motor shaft, centred within ±0.5 mm

## 11.3 Thermal Management

- Board is designed to use bottom copper pour as heatsink interface to motor aluminium frame

- Apply thermal pad or thermal paste between PCB bottom and aluminium frame — do not mount dry

- NTC thermistor reading is available on PA5 — implement firmware shutdown at 80 C

- Derate continuous phase current by 0.1 A per degree C above 40 C ambient

- Do not operate above 5 A continuous without confirming thermal coupling to aluminium frame

## 11.4 Logic Interface

CAUTION — All GPIO pins on STM32G431 are 3.3 V logic. Connecting 5 V signals to any GPIO, SWD, or UART pin will permanently damage the MCU. If using a 5 V USB-UART adapter, use a 3.3 V compatible adapter or add a voltage divider on the TX line.

## 11.5 Firmware Requirements

The following must be implemented in firmware before the board is safe to operate with a motor:

- Current sense offset calibration at startup (sample 1000x, average, store as zero offset)

- Phase overcurrent fault: disable all PWM if any phase current exceeds 6 A

- Bus overvoltage fault: disable all PWM if bus voltage exceeds 27 V

- Bus undervoltage fault: disable all PWM if bus voltage drops below 16 V

- Temperature fault: disable all PWM if NTC reads above 80 C

- Dead time: minimum 200 ns configured in TIM1 BDTR register

- Do not set all three high-side PWM outputs HIGH simultaneously at any time

# 12. Revision History

| Rev | Date | Author | Changes |
| --- | --- | --- | --- |
| 1 | Apr 2026 | MEKATRA | Initial release. 24 V, 5 A cont, STM32G431CBT6, UCC27710, TPS54334, INA181A1, IPZ40N04S5L2R8. |

SastaDrive Rev 1  |  MEKATRA  |  April 2026  |  This document is confidential and intended for internal engineering use only.