SastaDrive

Safety Reference & PCB Marking Guide

Rev 1  |  April 2026  |  MEKATRA

SCOPE:  This document defines all safety warnings, labels, and instructional text for the SastaDrive PCB silkscreen (back copper/silkscreen layer) and KiCad schematic notes frame. These texts are binding safety requirements — not cosmetic. Every label listed for the PCB must appear on the manufactured board. Every note listed for the schematic must appear on the relevant sheet before PCB release.

# PART 1 — PCB Silkscreen Text (Back of Board)

The following text must be printed on the back silkscreen layer of the SastaDrive PCB. Text is grouped by physical zone. Font size in KiCad should be 0.8 mm height minimum for all warning text, 1.0 mm for the board identity block. Do not reduce size to fit — reduce spacing between groups instead.

## 1.1  Board Identity Block  —  Centre of Board

Place this block in the geographic centre of the back silkscreen. This is the first thing anyone picking up the board reads.

SastaDrive  Rev.1

      3-Phase FOC Motor Driver

      24V / 5A CONT / 10A PEAK

      MEKATRA  |  mekatra.in

      READ DATASHEET BEFORE USE

Note for KiCad: Use F.Cu or B.Cu text layer for the board name line if you want it to be copper-etched (more permanent). Use B.Silkscreen for all remaining lines.

## 1.2  Power Input Zone  —  Near J1 (XT30 Connector)

Place immediately adjacent to the power input connector. Use 1.0 mm text height for the voltage/current line and 0.8 mm for the rest.

! VIN: 18V MIN  24V NOM  26V MAX

      ! DO NOT REVERSE POLARITY

      ! NO ONBOARD FUSE — FUSE EXTERNALLY

      ! NO REVERSE POLARITY PROTECTION

        DISCHARGE CAPS BEFORE HANDLING

        +   [PIN 1]   -   [PIN 2]

KiCad NOTE:  The '!' prefix on each line is intentional — it acts as a visual alarm character in monospace silkscreen. Include it exactly as shown. The pin polarity line must be adjacent to the connector pins themselves, not just in this block.

## 1.3  Motor Phase Output Zone  —  Near J2

MOTOR OUT: A  B  C

      FLOATING PHASE = HIGH VOLTAGE

      DO NOT TOUCH WHEN ACTIVE

      DISCONNECT POWER BEFORE WIRING

      DIRECTION: SWAP ANY 2 PHASES IN FW

## 1.4  Gate Driver / MOSFET Zone  —  Near Power Stage

Place near the UCC27710 ICs or near the MOSFET bank. This zone gets hot.

! HIGH VOLTAGE SWITCHING ZONE

      ! DO NOT PROBE PHASE NODE LIVE

      ! VCC = 12V — DO NOT SHORT

        MOSFET: IPZ40N04  40V 40A

        HOT SURFACE DURING OPERATION

## 1.5  Current Sense Zone  —  Near Shunt Resistors / INA181

ISENSE: 20mR / GAIN 20 / 5A FS

      CALIBRATE OFFSET AT EVERY BOOT

      ADC SAT AT >8.25A BUS CURRENT

      3.3V LOGIC — DO NOT APPLY 5V

## 1.6  MCU and Debug Zone  —  Near STM32 / SWD Header

STM32G431CBT6  |  3.3V ONLY

      SWD: SWDIO SWDCLK 3V3 GND

      UART: TX RX GND  |  115200 8N1

      ! GPIO MAX 3.3V — NO 5V SIGNALS

      BOOT0 LOW = NORMAL  HIGH = DFU

## 1.7  Encoder Header  —  Near J5

ENCODER: 3V3  GND  SCL  SDA

      I2C 400kHz  |  AS5600 ONLY

      MAGNET: 6mm DIAMETRIC ON SHAFT

      CENTRE WITHIN 0.5mm AXIALLY

## 1.8  Thermal and Mounting Zone  —  Board Edge or Near Mounting Holes

MOUNT TO ALUMINIUM FRAME

      APPLY THERMAL PAD — DO NOT MOUNT DRY

      MAX PCB SURFACE TEMP: 85C

      DERATE 0.1A/C ABOVE 40C AMBIENT

      NTC ON PA5 — IMPL SHUTDOWN AT 80C

## 1.9  Absolute Limits Strip  —  Long Edge of Board

This single-line strip runs along one long edge of the PCB. Keep it to 0.8 mm text height. It is the last-resort reminder for anyone who has lost the datasheet.

VIN MAX 26V | IPHASE MAX 10A | VGPIO MAX 3.3V | TAMB MAX 65C | NO REVERSE POLARITY | FUSE EXTERNALLY | SEE DATASHEET

## 1.10  Documentation Reference  —  Near Board Identity Block

Place a KiCad QR code or plain URL immediately below the identity block. This gives any future user a direct path to the datasheet.

[QR CODE HERE]

      DOCS: mekatra.in/sastadrive

      OR SCAN FOR DATASHEET

KiCad NOTE:  In KiCad use Add > QR Code from the footprint library, or the 'qrcode' footprint plugin. Set the URL to your documentation page. Place at minimum 5 mm from board edge. Use 0.2 mm module size for readability after manufacture.

# PART 2 — Schematic Safety Notes  (KiCad Text Blocks per Sheet)

Each note block below must be added to the corresponding KiCad schematic sheet using the Add Text tool. Use a distinct background colour fill where KiCad version supports it. Font size: 1.5 mm for warnings, 1.2 mm for standard notes. Notes are mandatory before PCB release — treat them as part of the design, not commentary.

KiCad PROCEDURE:  In KiCad 9, select Place > Add Text Box and set fill colour for each box. Group the text box with a nearby title using the G key so they move together. Use a red frame (line colour = #C0392B) for all WARNING blocks and an orange frame (#D35400) for all CAUTION blocks.

## 2.1  Sheet 1 — Top Level Hierarchy

Place in empty area, top-right corner of sheet. This is the overview warning that any engineer opening the project sees first.

WARNING — READ BEFORE EDITING OR MANUFACTURING

      ━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━

      Board: SastaDrive Rev 1  |  24V / 5A Cont / 10A Peak

      MCU: STM32G431CBT6  |  All GPIO = 3.3V MAX, NOT 5V TOLERANT

      Driver IC: UCC27710  |  VCC = 12V rail — never connect to 3.3V or 5V

      MOSFETs: IPZ40N04S5L2R8  |  Vds max 40V, Id max 40A — derated to 10A in application

      Current sense calibration MUST run at every firmware boot before PWM enable.

      Firmware MUST implement: overcurrent >6A, overvoltage >27V, undervoltage <16V, overtemp >80C.

      No crowbar, fuse, or reverse polarity protection on board — protect externally.

      Do not modify subsheets without reviewing inter-sheet net dependencies.

## 2.2  Sheet 2 — Controller (STM32 + Sensing + Power Supply)

2.2.1  Near STM32G431

WARNING — GPIO VOLTAGE LIMIT

      All STM32G431 GPIO pins are 3.3V max. NOT 5V tolerant.

      Connecting 5V signals to any pin (SWD, UART, GPIO) will permanently destroy the MCU.

      UART: Use 3.3V USB-UART adapter ONLY. If using 5V adapter, add 1k/2k divider on RX line.

      SWD: ST-LinkV2 must be set to 3.3V target voltage. Check VTref connection.

2.2.2  Near BOOT0 Switch

CAUTION — BOOT0 CONFIGURATION

      BOOT0 = LOW (pulled to GND via R51 10k) = Normal flash boot. DEFAULT STATE.

      BOOT0 = HIGH (press SW4) = DFU Bootloader mode. Use for firmware recovery only.

      Do not hold SW4 during normal operation — MCU will not run user firmware.

2.2.3  Near DC Bus Voltage Sense (R40/R41 Divider)

CAUTION — DC BUS VOLTAGE SENSE

      Divider: R40=100k, R41=10k. Scale factor = 10/(100+10) = 0.0909.

      ADC reading at 24V bus = 2.18V. ADC reading at 28V = 2.54V. Safe headroom exists.

      Firmware conversion: Vbus = ADC_voltage / 0.0909.

      Zener on ADC pin clamps at 3.3V. Do not remove zener — unprotected ADC pin.

2.2.4  Near DC Bus Current Sense (R44/R45 + U6)

WARNING — BUS CURRENT ADC SATURATION

      Shunt = 20mOhm. INA181A1 gain = 20. Full scale = 3.3V / (0.020 x 20) = 8.25A.

      ADC SATURATES ABOVE 8.25A. Firmware current limit must be set to 7A maximum.

      This sense is UNIDIRECTIONAL (REF = GND). Cannot measure regenerative current.

      Firmware conversion: Ibus = ADC_voltage / (0.020 x 20) = ADC_voltage x 2.5.

2.2.5  Near 5V to 3.3V LDO (IC2 MCP1700T)

CAUTION — 3.3V RAIL CURRENT LIMIT

      MCP1700T max output current = 250mA. Board draws ~100mA typical.

      Do not power external devices from the 3.3V header pin beyond 50mA.

      Input to LDO = 5V from TPS54334. If 5V rail fails, MCU loses power while gate drivers remain active.

      Firmware should detect PGOOD from 5V buck if possible; disable PWM on loss.

2.2.6  Near REF3033 (U2)

NOTE — ADC VOLTAGE REFERENCE

      REF3033 provides 3.300V ±0.2% precision reference to STM32 VREF+ pin.

      All ADC readings are referenced to this voltage, not to the 3.3V supply rail.

      Firmware must use VREF = 3.300V in all ADC conversion calculations, not 3.3V.

      100nF + 1uF decoupling on VREF+ pin. Do not remove — ADC noise will increase.

## 2.3  Sheet 3 — Active Inverter (Power Stage)

2.3.1  Main Sheet Warning  —  Top of Sheet 3

WARNING — HIGH VOLTAGE SWITCHING CIRCUIT

      ━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━

      Phase nodes switch between 0V and 24V at up to 40kHz. DO NOT PROBE LIVE PHASE NODES with standard oscilloscope probes without a high-voltage differential probe.

      Bootstrap capacitors hold charge after power-off. Wait 30 seconds after disconnect before touching board.

      All 6 MOSFETs must be confirmed present and correctly oriented before applying power. Missing or reversed FET = immediate shoot-through = board destruction.

      Gate resistors (R5/R6, R7/R8, R9/R10) must not be omitted or shorted. Removal causes gate ringing and shoot-through.

      COM pin of each UCC27710 connects to LOW-SIDE MOSFET SOURCE NODE, not AGND or PGND symbol.

2.3.2  Near Each UCC27710 (Place once per phase — 3 total)

CAUTION — UCC27710 GATE DRIVER

      VCC/VDD supply = 12V ONLY. Min 10V, Max 20V. Do not connect to 5V or 3.3V.

      HI and LI inputs are 3.3V logic compatible. Max input voltage = VCC. Do not float inputs.

      HB pin = bootstrap high side supply. Must not be connected to fixed supply rail.

      HS pin = floating phase node. Voltage swings 0V to 24V + bootstrap. Never ground.

      Dead time is NOT internal to UCC27710. It is set by STM32 TIM1 BDTR register. Min 200ns recommended.

2.3.3  Near Each Phase Shunt Resistor (R27, R35, R51)

NOTE — CURRENT SENSE SHUNT

      Shunt = 20mOhm 1% 2W (HOYLR2512-2W-20MR). Power dissipation at 5A = 0.5W — within rating.

      Sense taps must connect at resistor pads only (Kelvin sense). DO NOT branch off the power trace.

      1nF differential cap must be placed across INA181 IN+ and IN- within 3mm of IC.

      INA181 REF pin must connect to 1.65V midpoint (VREF/2 from 10k/10k divider on 3.3V).

2.3.4  Near Each INA181 (U3, U4, U5)

NOTE — INA181A1 CURRENT SENSE AMPLIFIER

      Gain = 20 V/V (INA181A1). Do not substitute INA181A2 (gain=50) or INA181A3 (gain=100) — ADC will saturate.

      Supply = 3.3V. Common mode input = 0V to 80V (24V bus well within range).

      Output at 0A = 1.65V (REF pin voltage). Output at +5A = 3.65V (clipped by supply).

      Firmware must subtract offset measured at boot. Run calibrate() before enabling PWM.

      Firmware conversion: I = (Vout - Vref) / (Rshunt x Gain) = (Vout - 1.65) / (0.020 x 20).

2.3.5  Near Bootstrap Circuit (D3/D4/D5 + C9/C21/C25)

CAUTION — BOOTSTRAP SUPPLY

      Bootstrap diode = SS14 (1A Schottky, 40V). DO NOT substitute 1N4148 — insufficient current, wrong Vf.

      Bootstrap cap = 220nF 10V minimum rating ceramic. Do not use electrolytic.

      High-side cannot turn ON if low-side has never conducted (bootstrap cap not charged).

      Firmware must run low-side pre-charge pulse sequence before first high-side enable.

2.3.6  Near Buck Converter IC1 (TPS54334)

WARNING — TPS54334 INPUT VOLTAGE LIMIT

      TPS54334 absolute maximum VIN = 28V. Nominal bus = 24V. Margin = 4V only.

      Any load dump or supply transient above 28V will permanently damage this IC.

      TVS diode (SMBJ26A) on VIN is mandatory — do not omit or substitute higher clamp voltage.

      Output = 12V for UCC27710 VCC. Verify output with multimeter before connecting gate drivers.

      PGOOD pin indicates valid output. Monitor in firmware if connected to GPIO.

## 2.4  Sheet 4 — 12V to 5V Buck Converter

2.4.1  Main Sheet Note  —  Top of Sheet 4

CAUTION — 5V BUCK CONVERTER (TPS54334)

      Input = 12V from first stage buck. Output = 5V for MCP1700T LDO input.

      FB resistor values set output voltage. R56/R58 must be calculated and fitted — do not leave unvalued.

      Vout = 0.8 x (1 + R56/R58). For 5V: use R56=100k, R58=19.1k. Verify with multimeter after assembly.

      Compensation cap C47 must be calculated — do not leave as 'some pF'. Typical = 47pF for this configuration.

      Output caps C45/C46: Use 22uF 10V ceramic each. Total output cap = 44uF minimum.

      Inductor L2 = ASPI-0630LR-220M-T15 = 22uH. Verify saturation current >= 3A.

      If 5V rail is absent, MCU loses power. PWM may remain active from gate driver side. Design risk — noted.

# PART 3 — Firmware Safety Requirements  (Reference for Developer)

This section defines the minimum firmware behaviours that must be implemented before the board is safe to operate with a motor connected. These are not optional features — they are safety requirements. Add this list as a text block on Sheet 1 of the schematic and include it in any firmware README.

The following table defines the minimum requirements. A checkmark against each item is required before FOC enable is considered safe.

| # | Requirement | Threshold / Value | Action on Fault |
| --- | --- | --- | --- |
| 1 | Current sense offset calibration at boot | Run before any PWM enable. Sample 1000x, average. | Block PWM enable until complete |
| 2 | Phase overcurrent protection — Phase A | > 6A instantaneous | Disable all 6 PWM outputs immediately. Latch fault. |
| 3 | Phase overcurrent protection — Phase B | > 6A instantaneous | Disable all 6 PWM outputs immediately. Latch fault. |
| 4 | Phase overcurrent protection — Phase C | > 6A instantaneous | Disable all 6 PWM outputs immediately. Latch fault. |
| 5 | DC bus overvoltage protection | > 27V on DCBUS_VSENSE | Disable all PWM. Latch fault. Log event. |
| 6 | DC bus undervoltage protection | < 16V on DCBUS_VSENSE | Disable all PWM. Latch fault. Do not attempt restart. |
| 7 | Bus current ADC saturation guard | > 7A on INPUT_CSENSE | Reduce current demand. Fault at 8A. |
| 8 | NTC over-temperature protection | > 80C on PA5 | Disable PWM. Do not re-enable until < 60C. |
| 9 | Dead time configuration (TIM1 BDTR) | Minimum 200ns, recommended 300ns | Must be set before first PWM enable. |
| 10 | Centre-aligned PWM mode (TIM1) | CMS bits = 01 or 10 or 11 | Required for valley sampling of ADC. |
| 11 | ADC triggered at PWM valley | TIM1 trigger to ADC JSWSTART | Do not use software trigger for current sense. |
| 12 | Bootstrap pre-charge sequence | Low-side ON for 100us before first high-side enable | Without this, high-side FET will not turn on. |
| 13 | All-high-side shoot-through guard | Never set all 3 high-side outputs HIGH simultaneously | Hardware interlock via TIM1 — verify in code review. |
| 14 | Encoder read validity check | AS5600 AGC value in valid range (0–255) | Do not enter FOC closed-loop if encoder read fails. |
| 15 | Soft start on enable | Ramp current demand from 0 to target over 50ms min | Prevents inrush on enable. Required for motor safety. |
| 16 | Watchdog timer | IWDG or WWDG active during motor run | Reset system if firmware hangs during active drive. |

SastaDrive Rev 1  |  MEKATRA  |  April 2026     This document must be reviewed and signed off by the lead engineer before PCB manufacture.